Blowin’ in the Wind Teachers Notes

This topic breaks into two parts.

Part One explores how we model and how tables, graphs and a background knowledge of direct proportion can help our thought and investigative processes.

Part Two exposes the student to the various formulae used in this field. It would be better if the student did not have these formulae for part one.

The formula for the area of a circle is worked out by experiment. This is to expose the student to the process of gathering data, charting data, making conjectures as to the type of model to use, hunting for the transformation that will produce a straight line graph through the origin, concluding direct proportion and finding the constant of proportion.

It shouldn’t matter if the student already has the knowledge A = πr2. It is the investigative process which is being revealed.

# Relationships

When a wind generator is designed various factors are considered

- The potential windiness of the site

This aspect is not examined in the maths; perhaps this would be a good link to the Geography department.

- The size of the blades

- The number of generators require on the site to guarantee a certain output.

# Relationship 1

How is the area swept out by the arm related to its length?

Students can count squares and make use of symmetry to quarter the work; working from the smallest circle outwards will also reduce duplication of effort.

Some discussion that 1 cm2 = 100 mm2 might have to take place … deduced from the graph paper … and a reminder that area entered in the table is the number of squares counted

divided by 100.

| radius(cm) | area(cm2) |
| --- | --- |
| 0 | 0.00 |
| 0.5 | 0.79 |
| 1 | 3.14 |
| 1.5 | 7.07 |
| 2 | 12.57 |
| 2.5 | 19.63 |
| 3 | 28.27 |
| 3.5 | 38.48 |

(c)	Draw a graph of the data.

radius v area of circle

40.00

35.00

30.00

Area (cm2)

25.00

20.00

15.00

10.00

5.00

0.00

0	0.5	1	1.5	2	2.5	3	3.5

radius (cm)

This chart has been drawn using Excel which is a speedy and pleasing way of generating informative charts.

If possible the student could, as a precursor to this investigation, explore the graphs associated with squares, cubes, power of 4 etc

looking for common features and differences.

squares of numbers

Cubes of numbers

30	140

25	120

100

20

80

15

60

10

40

5	20

0

0	1	2	3	4	5

numbers

0

0	1	2

numbers

3	4	5

power 4

power 5

700

3500

600

3000

500

2500

400

2000

300

1500

200

1000

100

500

0

0	1	2	3	4	5

number

0

0	1	2

number

3	4	5

The radius vs area graph suggests we explore the powers of the radius …

so starting with the square of the radius we get the following table and its corresponding chart.

We immediately see a straight line graph passing through the origin.

| Radius2 (cm) | Area (cm2) |
| --- | --- |
| 0 | 0.00 |
| 0.25 | 0.79 |
| 1 | 3.14 |
| 2.25 | 7.07 |
| 4 | 12.57 |
| 6.25 | 19.63 |
| 9 | 28.27 |
| 12.25 | 38.48 |

40.00

35.00

30.00

25.00

20.00

15.00

Area v radius squared

(iii)	So A = r2 times a constant.

10.00

5.00

0.00

0    1    2    3    4    5    6    7    8    9

radius squared (cm2)

10  11  12

Use your table and graph to find the value of the constant.

At this point various questions arise. Why have we omitted the (0, 0) case?

Presuming the student is working with their own ‘rough’ data, the values of  the constant may vary a little … worth a discussion … does that mean it’s not constant?

So how would one express the relationship between radius and area swept through? A = 3·14r2 is accurate enough for most cases … but is it exact?

π can be introduced if not already known.

| Radius2 (cm) | Area (cm2) | r2/A |
| --- | --- | --- |
| 0 | 0.00 |  |
| 0.25 | 0.79 | 3.14 |
| 1 | 3.14 | 3.14 |
| 2.25 | 7.07 | 3.14 |
| 4 | 12.57 | 3.14 |
| 6.25 | 19.63 | 3.14 |
| 9 | 28.27 | 3.14 |
| 12.25 | 38.48 | 3.14 |

(e)	Would two turbines with arm length 4 metres sweep out the same area as a turbine with arm length 8 metres?

This question has been placed here because most students find the actual answer counter- intuitive. It also helps to illustrate the meaning of “area is proportional to the square of the radius” and allows a discussion of the affect changing one variable has on the value of another … double the radius and get four time the area.

There is also the possibility of looking at cost and value for money. Would 4 small ones cost the same as a big one?

Supplementary question

# Relationship 1a

If you look at a wind turbine turning it looks so slow and harmless.

How fast is the tip of a turbine’s blade moving?

The supplementary question is included in case you wish to develop the formula for circumference in the same context. It is also a bit of a surprise that something which seems to move at a gentle pace is actually moving at a speed that would do you damage!

e.g. If a blade is 50 metres long and it makes one revolution in 7 seconds … nice and slow

… the tip of the blade is travelling at just over 100 miles an hour.

Taking the same approach as when looking for the other relationships, we find obviously that this leads directly to the straight line.

This provides another reason for exploring blade tip speed.

It may be of interest to note that the noise emissions from the blades is directly proportional

to the 5th power of blade speed … a small increase in tip speed can ma difference in the noise.

e a large

# Relationship 2

The table below gives the energy in the wind at different speeds for a turbine with a 10 m2 sweep.

| Wind speed (Metres per second) | Wind energy (approx) (Joules per metre2) |
| --- | --- |
| 2 (light breeze) | 5 |
| 5 (Gentle breeze) | 76 |
| 10 (Fresh breeze) | 610 |
| 15 (Near gale) | 2060 |
| 20 (Fresh gale) | 4880 |
| 25 (Strong gale) | 9530 |

1	(i)	Draw a graph of wind speed v wind energy

| Wind speed (m/s) |  | Wind energy (joules/m2) |
| --- | --- | --- |
| 2 | 5 |  |
| 5 | 76 |  |
| 10 | 610 |  |
| 15 | 2060 |  |
| 20 | 4880 |  |
| 25 | 9530 |  |

10000

9000

8000

7000

6000

5000

4000

3000

2000

1000

0

Wind speed v Wind energy

0	5	10

15	20	25

wind speed (m/s)

The graph suggests we should look at a power of wind speed. Try wind speed squared …

| Wind speed squared | W ( | ind energy joules/m2) |
| --- | --- | --- |
| 4 | 5 |  |
| 25 | 76 |  |
| 100 | 610 |  |
| 225 | 2060 |  |
| 400 | 4880 |  |
| 625 | 9530 |  |

10000

9000

8000

7000

6000

5000

4000

3000

2000

1000

0

Wind speed2 v Wind Energy

0	200	400

wind speed2

600

…	not a straight line. Try ‘cubed’.

[On a spreadsheet this procedure is greatly eased as you only have to change the index in the windspeed column and the graph will automatically change. Thereafter edit the labels in the graph to agree with the power being illustrated]

| Wind speed cubed | Win (jo | d energy ules/m2) |
| --- | --- | --- |
| 8 | 5 |  |
| 125 | 76 |  |
| 1000 | 610 |  |
| 3375 | 2060 |  |
| 8000 | 4880 |  |
| 15625 | 9530 |  |

10000

9000

8000

7000

6000

5000

4000

3000

2000

1000

0

Wind speed3 v Wind Energy

0	2500    5000    7500   10000  12500  15000

wind speed3

Here we do get a straight line graph passing through the origin.

Wind energy is directly proportional to the cube of the wind speed.

E = ku3…	and the next question … what is the value of k?

| Wind speed cubed | Wind energy (joules/m2) | E/u3 |
| --- | --- | --- |
| 8 | 5 | 1.6 |
| 125 | 76 | 1.644736842 |
| 1000 | 610 | 1.639344262 |
| 3375 | 2060 | 1.638349515 |
| 8000 | 4880 | 1.639344262 |
| 15625 | 9530 | 1.639559286 |

A table of values suggests that it is 1·64

# Conclusion

For a wind turbine with a 10 m2 sweep E = 1·64u3 where E is the wind energy and u is the wind speed in m/s.

# Relationship 3

This table shows how different wind energies produce different power (Watts) [from a wind turbine with a 10 square metre area of sweep]

| Wind energy (Joules per metre2) | Power (Watts) |
| --- | --- |
| 5 | 30 |
| 76 | 460 |
| 610 | 3700 |
| 2060 | 12400 |
| 4880 | 29300 |
| 9530 | 57200 |

- Draw a graph and work out the relationship between Energy and Power.

Energy v power

60000

50000

40000

30000

20000

10000

0

0	2000	4000	6000	8000

Energy (joules/m2  )

The straight line graph passing through the origin tells us that energy is directly proportional to power …	E = kP … a quick check gives us k = 1/6

So E = 1/6P

- Relate wind speed to power.

We now have E = 1·64u3 and E = 1/6P for a turbine with a 10 m2 sweep. Thus

164u3  P

## 6

P 9 84u3

- Use your findings to help you work out the energy and power in the wind at the following wind speeds:

## 

This, and the exercise which follows is meant to add purpose and bring back the daily context in which the student will encounter the topic.

| speed (m/s) | 4 | 8 | 12 | 18 | 24 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| energy (joules/m2) | 104.96 | 839.68 | 2833.92 | 9564.48 | 22671.36 | 44280 |
| power (watts) | 629.76 | 5038.08 | 17003.52 | 57386.88 | 136028.16 | 265680 |

# Using the power.

A toaster has a power

rating of 600 watts

An ordinary light bulb has a power rating of 60 watts

An electric fire has a power

rating of 3000 watts

An electric kettle has a power rating of 1000 watts

- If you had a wind turbine with an area of sweep of 10 square metres -

  - How many light bulbs could you power with a gentle breeze?

  - How many light bulbs could you power with a fresh breeze?

  - How many toasters could you power with a gentle breeze?

  - What sort of wind would you need to power an electric kettle?

  - What sort of wind would you need to power an electric fire?

- If you go round your house and look at the electrical appliances you will find somewhere on them the power rating (Watts) each one needs.

Make up a table of 10 most used appliances in your house.

| Appliance | Power rating (Watts |
| --- | --- |
| light bulb | 60 |
| kettle | 1000 |
| toaster | 600 |
| fire | 3000 |
| TV |  |

Try to explain why you would need a wind turbine with an area of sweep greater than 10 square metres to meet all your power needs. Discuss your findings with others. What conclusion do you come to?

# Formulae … Being efficient.

If the turbine took none of the wind’s energy away, it would be just as windy behind the turbine as in front of it.

## 0% efficient

If the wind turbine could extract all the energy from the wind then the air would be totally still immediately behind the turbine … having no energy left.

## 100% efficient

We know this doesn’t happen. The wind turbine can only extract a percentage of the energy available.

This percentage is called the efficiency of the turbine. The theoretical maximum for this is 59%.

This is known as the Betz limit.

# Supplementary Question.

What would you not be allowed to do anywhere close to a wind turbine?

The kite is used as a graphic illustration of before and after. It is not meant to be an inspiration to practice. However, who knows? It is felt safer to draw to the student’s attention how irresponsible the person in the diagram is being!

# The proof of the pudding is in the eating.

Having explored the construction of a model we would like to consider where it falls short of reality. Galileo used a mathematical model that showed that a hammer and a feather should fall to the earth at the same time (circa 1616). Air resistance put paid to that model. It wasn’t until Apollo 15 (1971) that we saw the model working … on the moon.

The next part gives the student the formulae that are currently used by engineers in this field. The student should be encouraged to see how well the formulae fit their own derived models.

# Formulae

Expose the student to the ‘used’ model, to the idea of efficiency and to the source of the data from which we derived our model.

- Area swept = πr2 where r is the arm length of the blade.

- Energy in wind = 1/2 u3d where u is the wind speed in m/s and d is the air  density measured in kg/m3.

- Power = Energy Area Betz limit.

Now is the point to have class discussion as to from where the discrepancies between  our model and (a) the engineer’s model and (b) reality come.

# Exercise

Write out a formula for calculating the power P = 0·5 0·59 u3d πr2.

P = 0·927 u3d r2

The questions which follow are aimed at giving the student the feel for the formula and a chance to practice substitution, manipulation and solving equations.

I’ll leave the answers and any extension of the exercise to the user.

As an extension to the topic one might like to address the source of the data.

How are these formulae derived? On what logic are they based?

How does one determine the speed of the wind? How does one determine the efficiency?

The internet is a good source of  this sort of information.

# Further Notes

- Above are listed the main outcomes and experiences which affect the project.

- I’ve recommended 4 separate periods for this. These may or may not be sequential. The principal objective is to create mathematical models all of which depend on ‘transforming’ curves until a straight line passing through the origin is obtained.

- I really recommend the use of technology when making tables, graphs and conjectures while hunting for a model. Understanding that direct proportion is a deduction when a straight line passing through the origin is obtained, is more important than number crunching and making hand-drawn graphs.

- The project exposes the student to direct proportion

  - between one quantity and another

  - between one quantity and the square of another.

  - between one quantity and the cube of another.

It also tries to indicate why the student/scientist is likely to go hunting for such	a model. Organising a lesson before the project launches involving drawing graphs on Excel will pay dividends. Students should be aware of the graphs of y = xn, recognizing the common features and also the differences as n increases.

This might best be achieved by drawing all the graphs on the same diagram. e.g.

Powers of x

120

110

100

90

80

70

60

50

40

30

20

10

0

0

x x2 x3 x4

1	2	3	4	5

x

It may be of interest to note that the noise emissions from the blades is directly proportional

to the 5th power of blade speed … a small increase in tip speed can ma difference in the noise.

e a large

- The more academic sttudent should be asked to consider the following: “If we have a graph of y against x and it produces the curve associated with x2 then drawing a graph of y against x2 will produce a straight line.”

- Visit these websites to find out more about wind speed and weather conditions:

http://www.stormfax.com/beaufort.htm

http://en.wikipedia.org/wiki/Beaufort_scale

http://www.renewableuk.com/page/UKWEDhome

www.ecotricity.co.uk/windenergy/

www.urbanwindenergy.org.uk/

www.yes2wind.com/